Team 11

Problem Definition: Design an Electronic Voting Machine (EVM) Using ATMega32 Microcontroller for secure and efficient voting, featuring user-friendly interface, candidate selection, vote counting, and result display, aimed at enhancing the voting process in elections.

Team Details:

Amith A - 319 (01FE22BCS116)

Ganesh N – 336 (01FE22BCS194)

Gautam N – 342 (01FE22BCS219)

Appu S C – 355 (01FE22BCS274)

#define F_CPU 8000000UL            /* Defining CPU Frequency e.g. here 8MHz */

#include <avr/io.h>                /* Including AVR std. library file */

#include <util/delay.h>            /* Including inbuilt defined Delay header file */

#include <stdio.h>

#include <string.h>

#define LCD_Data_Dir DDRD          /* Defining LCD data port direction */

#define LCD_Data_Port PORTD        /* Defining LCD data port */

#define LCD_Command_Dir DDRB       /* Defining LCD command port direction register */

#define LCD_Command_Port PORTB     /* Defining LCD command port */

#define RS PB0                     /* Defining Register Select (data/command reg.) pin */

#define RW PB1                     /* Defining Read/Write signal pin */

#define EN PB2                     /* Defining Enable signal pin */

#define KEY_PRT PORTA              // keyboard PORT

#define KEY_DDR DDRA               // keyboard DDR

#define KEY_PIN PINA               // keyboard PIN

unsigned char keypad[4][4] = {

{'7', '8', '9', '/'},

{'4', '5', '6', '*'},

{'1', '2', '3', '-'},

{'C', '0', '=', '+'}

};

unsigned char colloc, rowloc;

unsigned int votes[4] = {0, 0, 0, 0}; // Votes for 4 candidates

unsigned int voter_count = 0;

void LCD_Command(unsigned char cmnd) {

LCD_Data_Port = cmnd;

LCD_Command_Port &= ~(1 << RS);  /* RS=0 command reg. */

LCD_Command_Port &= ~(1 << RW);  /* RW=0 Write operation */

LCD_Command_Port |= (1 << EN);   /* Enable pulse */

_delay_us(1);

LCD_Command_Port &= ~(1 << EN);

_delay_ms(3);

}

void LCD_Char(unsigned char char_data) {  /* LCD data write function */

LCD_Data_Port = char_data;

LCD_Command_Port |= (1 << RS);  /* RS=1 Data reg. */

LCD_Command_Port &= ~(1 << RW); /* RW=0 write operation */

LCD_Command_Port |= (1 << EN);  /* Enable Pulse */

_delay_us(1);

LCD_Command_Port &= ~(1 << EN);

_delay_ms(1);

}

void LCD_Init(void) {                /* LCD Initialize function */

LCD_Command_Dir = 0xFF;          /* Make LCD command port direction as o/p */

LCD_Data_Dir = 0xFF;             /* Make LCD data port direction as o/p */

_delay_ms(20);                   /* LCD Power ON delay always >15ms */

LCD_Command(0x38);               /* Initialization of 16X2 LCD in 8bit mode */

LCD_Command(0x0C);               /* Display ON Cursor OFF */

LCD_Command(0x01);               /* Clear display */

LCD_Command(0x80);               /* Cursor at home position */

}

void LCD_String(char *str) {         /* Send string to LCD function */

int i = 0;

while (str[i] != 0) {            /* Send each char of string till the NULL */

LCD_Char(str[i]);

i++;

}

}

void LCD_Clear() {

LCD_Command(0x01);               /* Clear display */

LCD_Command(0x80);               /* Cursor at home position */

}

char keyfind() {

while (1) {

KEY_DDR = 0xF0;              /* set port direction as input-output */

KEY_PRT = 0xFF;

/* Check for pressed key in each row */

for (int row = 0; row < 4; row++) {

KEY_PRT = ~(0x10 << row); /* Set row to low */

_delay_us(1);             /* Add small delay for debounce */

colloc = (KEY_PIN & 0x0F); /* Read columns */

if (colloc != 0x0F) {

rowloc = row;

goto find_col;        /* Exit from loop */

}

}

}

find_col:

if (colloc == 0x0E) return keypad[rowloc][0];

if (colloc == 0x0D) return keypad[rowloc][1];

if (colloc == 0x0B) return keypad[rowloc][2];

return keypad[rowloc][3];

}

int check_pin() {

char entered_pin[5];

char correct_pin[5] = "4016";  // Set your 4-digit PIN here

char master_key[5] = "1234";   // Set your master key here

int i = 0;

LCD_Clear();

LCD_String("Enter PIN:");

LCD_Command(0xC0);

while (i < 4) {

entered_pin[i] = keyfind();

if (entered_pin[i] < '0' || entered_pin[i] > '9') {

LCD_Clear();

LCD_String("Invalid Char");

_delay_ms(500);

return -1;  // Return immediately if an invalid character is entered

}

LCD_Char('*'); // Display * for each digit entered

i++;

_delay_ms(100); // Delay to prevent multiple * being displayed

}

entered_pin[i] = '\0'; // Null-terminate the entered PIN

_delay_ms(100); // Small delay before checking PIN

if (strcmp(entered_pin, correct_pin) == 0) {

return 1;  // Correct voter PIN

} else if (strcmp(entered_pin, master_key) == 0) {

return 2;  // Correct master key

} else {

return 0;  // Incorrect PIN

}

}

void vote_counting() {

char key;

char vote;

LCD_Clear();

LCD_String("1:BJP 2:INC");

LCD_Command(0xC0);

LCD_String("3:AAP 4:JDS");

while (1) {

key = keyfind();

if (key >= '1' && key <= '4') {  /* Valid candidate */

vote = key;                  /* Store the selected candidate */

LCD_Clear();

LCD_String("Confirm: =");

_delay_ms(100);              /* Short delay for debounce */

while (keyfind() != '=');    /* Wait for confirmation */

votes[vote - '1']++;         /* Increment vote count */

voter_count++;

LCD_Clear();

LCD_String("Vote Recorded");

_delay_ms(200);

break;

}

}

}

void display_votes() {

char buffer1[16];

char buffer2[16];

LCD_Clear();

snprintf(buffer1, sizeof(buffer1), "BJP:%d INC:%d", votes[0], votes[1]);

snprintf(buffer2, sizeof(buffer2), "AAP:%d JDS:%d", votes[2], votes[3]);

LCD_Command(0x80); // Move cursor to the first line

LCD_String(buffer1);

LCD_Command(0xC0); // Move cursor to the second line

LCD_String(buffer2);

_delay_ms(300);

}

void display_winner() {

int max_votes = 0;

int winner = -1;

int max_vote_count = 0;

int equal_votes = 0;

for (int i = 0; i < 4; i++) {

if (votes[i] > max_votes) {

max_votes = votes[i];

winner = i;

}

}

for (int i = 0; i < 4; i++) {

if (votes[i] == max_votes) {

max_vote_count++;

}

}

if (max_vote_count == 4) {

equal_votes = 1;

}

LCD_Clear();

if (equal_votes) {

LCD_String("Equal Votes");

} else {

if (winner != -1) {

LCD_String("Winner:");

LCD_Command(0xC0);

switch (winner) {

case 0: LCD_String("BJP"); break;

case 1: LCD_String("INC"); break;

case 2: LCD_String("AAP"); break;

case 3: LCD_String("JDS"); break;

}

} else {

LCD_String("No Votes");

}

}

_delay_ms(300);

}

void master_function() {

char key;

LCD_Clear();

LCD_String("1:Show Votes");

LCD_Command(0xC0);

LCD_String("2:Show Winner");

while (1) {

key = keyfind();

if (key == '1') {

display_votes();

_delay_ms(400);

display_winner();

break;

} else if (key == '2') {

display_winner();

_delay_ms(300);

break;

}

}

}

int main(void) {

LCD_Init();

LCD_String("    TEAM_11");

_delay_ms(300);

LCD_Clear();

LCD_String("Every vote counts");

_delay_ms(300);

LCD_Clear();

LCD_String("   Vote wise");

_delay_ms(200);

LCD_Clear();

LCD_String(" Authentication:");

_delay_ms(200);

// Define the initial security key

char initial_key[5] = "7676";  // Set your initial security key here

char entered_key[5];

while (1) { // Infinite loop to handle continuous voting

// Check initial security key

LCD_Clear();

LCD_String("Enter Key:");

LCD_Command(0xC0);

// Loop to read the entered key

int i = 0;

while (i < 4) {

entered_key[i] = keyfind();

LCD_Char('*'); // Display * for each digit entered

i++;

_delay_ms(100); // Delay to prevent multiple * being displayed

}

entered_key[i] = '\0'; // Null-terminate the entered key

_delay_ms(100); // Small delay before checking the key

if (strcmp(entered_key, initial_key) != 0) {

LCD_Clear();

LCD_String("Wrong Key");

_delay_ms(500);

continue; // Restart the loop if the key is incorrect

}

// Key is correct, proceed to PIN entry and voting

int pin_check = check_pin();

if (pin_check == 1) {

vote_counting();

LCD_Clear();

LCD_String("Next Voter");

_delay_ms(500);

} else if (pin_check == 2) {

master_function();

break;  // Exit the loop after displaying master functions

} else if (pin_check == 0) {

LCD_Clear();

LCD_String("Invalid PIN");

_delay_ms(500);

}

}

while (1); // Infinite loop to stop execution

return 0;

}